HOW THE ROBOT WILL WORK:

- What will trigger the start and end of the program?

- GUI with start and stop button

- Shapes that will represent Start and Stop actions

- MAIN LOOP

2.1) Idle & Watch: The camera watches the PICK ZONE.

2.2) Item detected: (detector.py/vision_pub.py)

- Run YOLO-n  model on a downscaled frame (e.g., 320×320).

- Keep only detections whose centroid falls inside the PICK ZONE ROI (rectangle or polygon).

- Treat the item as “present” only if confidence stays above a threshold for a few consecutive frames (temporal debounce).

Minimal logic

- conf_thresh: 0.6 (start here; tune later)

- stable_frames: 3 (number of consecutive frames needed to confirm)

- cooldown_frames: 10–15 (after a pick, ignore residual detections until the area is empty again)

Pseudocode

# detector.py (sketch)

from collections import deque

import cv2

import numpy as np

from ultralytics import YOLO

model = YOLO("best-grocery.pt")

roi_poly = np.array([(x1,y1),(x2,y2),(x3,y3),(x4,y4)])  # PICK ZONE polygon in image pixels

def in_roi(cx, cy, poly):

return cv2.pointPolygonTest(poly, (float(cx), float(cy)), measureDist=False) >= 0

present_buffer = deque(maxlen=6)  # holds True/False for last frames

cooldown = 0

while True:

frame = get_frame_from_picam()               # BGR

small = cv2.resize(frame, (320, 320))        # keep >5 FPS

results = model.predict(small, verbose=False)[0]

in_zone = False

best = None  # (class, conf, bbox, centroid)

for b in results.boxes:

conf = float(b.conf[0])

cls  = int(b.cls[0])

x1,y1,x2,y2 = map(float, b.xyxy[0])

cx, cy = (x1+x2)/2, (y1+y2)/2

if conf >= 0.6 and in_roi(cx, cy, roi_poly):

in_zone = True

if (best is None) or (conf > best[1]):

best = (cls, conf, (x1,y1,x2,y2), (cx,cy))

# Debounce and cooldown

if cooldown > 0:

present_buffer.append(False)

cooldown -= 1

else:

present_buffer.append(in_zone)

confirmed_present = sum(present_buffer) >= 3  # stable for ≥3 of last frames

if confirmed_present and best:

# -> TRIGGER: item present

publish_item_detected(class_id=best[0], conf=best[1], centroid_px=best[3])

cooldown = 15        # block re-triggers until arm has started moving

present_buffer.clear()

# For re-arming after a pick:

# wait until present_buffer becomes all False for a few frames before accepting a new item

Notes

- If you always place one item centered, you can skip bounding boxes and just classify the center crop. But detector-first is more general and helps when the item is slightly off-center.

How the trigger fits your FSM

- IDLE → SEE_ITEM when confirmed_present == True (from A or A+B).

- Pass centroid_px to your calibration mapper → base coordinates.

- If class confidence ≥ threshold, continue; else show “low confidence” prompt and stay in SEE_ITEM.

- After gripper closes and the arm leaves the zone, set cooldown and require N empty frames before allowing the next trigger (prevents double-grabs).

2.3) Classify (detector.py)

- Input & crop

  - The Pi camera streams video.

  - We crop a fixed PICK ZONE ROI (rectangle/polygon) from each frame.

- Preprocess

  - Resize ROI to 224×224 (YOLOv8-cls default), normalize.

- Inference (YOLOv8n-cls)

  - Run the classification model (trained on your 6 grocery classes; optionally an “empty” class).

  - Output: class probabilities p(class|frame) and the top-1 class with its confidence.

- Temporal decision (smoothing)

  - A sliding window buffer over the last K frames (e.g., K=7) stabilizes the decision with a majority vote + confidence checks.

- Publish result

  - When the decision is “stable,” we publish an event to the supervisor:
{"state": "PRESENT", "class": "bottle", "conf": 0.86} (or {"state":"EMPTY"}).

Training (brief)

- Data layout (train – val - test)

- Command (example):
yolo task=classify mode=train model=yolov8n-cls.pt data=dataset imgsz=224 epochs=25 batch=32

- Export to Pi:
yolo export format=torchscript (or just copy best.pt and use Ultralytics Python API on the Pi).

How many frames to confirm the grocery type?

Use a two-gate rule: presence gate → class gate. This avoids flicker and wrong picks.

Logic

- We start filling a present_buffer with booleans “is this frame not-empty?”

- If at least 3 consecutive frames say “not-empty” → we consider an item present.

- Then we fill a class_buffer (top-1 labels only when conf ≥ 0.60).

- When ≥5 of the last 7 frames agree on the same class → confirm class and trigger the pick.

This keeps the robot from mis-classifying due to one noisy frame and still reacts in < 0.7 s.

How the robot knows empty vs not-empty Pick Zone

Option A — Train an “empty” class (simplest integration)

- Include an empty/ folder with many ROI crops of the Pick Zone when nothing is there (different lighting/background).

- During inference, check the top-1 class:

  - If top_class == "empty" and conf ≥ 0.60 for M frames (e.g., M=5/7) → state = EMPTY.

  - If top_class != "empty" and conf ≥ 0.60 with the presence/class rules above → state = PRESENT with that class.

- After a pick, require N empty frames (e.g., N=8/10) before re-arming; this prevents re-triggering on residual motion.

Pros: One model handles everything; no motion code needed.
Cons: You must collect enough empty examples under varying light so it generalizes.

Option B — Motion gate + classifier (lighter training)

- Use a background subtractor or simple frame differencing in the ROI:

  - If foreground area > threshold → we say “candidate present” and start/continue classifying.

  - If foreground area ≤ threshold for N frames → “EMPTY.”

- Combine with the same 3-of-N and 5-of-7 class stability rules.

Pros: No “empty” class to train; very responsive.
Cons: Sensitive to auto-exposure flicker (lock camera exposure/gain).

Practical tip: If you have time, do both: keep an “empty” class and still gate on motion to save compute and avoid false positives when light changes.

Pseudocode (classification with “empty” class)

# classify_roi.py

from collections import deque

from ultralytics import YOLO

import cv2, numpy as np

ROI = (x, y, w, h)

CONF_THRESH = 0.60

CLASS_WINDOW = 7

CLASS_REQUIRED = 5

PRESENCE_STABLE_FRAMES = 3

EMPTY_REQUIRED = 5

COOLDOWN_FRAMES = 12

model = YOLO("best_cls.pt")  # yolov8n-cls trained on 6+empty

class_buf = deque(maxlen=CLASS_WINDOW)

present_buf = deque(maxlen=PRESENCE_STABLE_FRAMES)

cooldown = 0

while True:

frame = get_frame()

roi = crop(frame, ROI)

probs = model(roi, verbose=False)[0].probs  # tensor of class probs

top_idx = int(probs.top1)

top_conf = float(probs.top1conf)

top_name = model.names[top_idx]

# Frame-level presence

is_not_empty = (top_name != "empty") and (top_conf >= CONF_THRESH)

if cooldown > 0:

cooldown -= 1

continue

present_buf.append(is_not_empty)

# Gate 1: presence stability

if sum(present_buf) >= PRESENCE_STABLE_FRAMES:

# Gate 2: class stability over last K frames

if top_conf >= CONF_THRESH:

class_buf.append(top_name)

if len(class_buf) == CLASS_WINDOW:

# majority vote

winner = max(set(class_buf), key=class_buf.count)

if winner != "empty" and class_buf.count(winner) >= CLASS_REQUIRED:

publish_present(winner)  # -> supervisor picks

cooldown = COOLDOWN_FRAMES

class_buf.clear()

present_buf.clear()

# Re-arm to EMPTY after a pick (done in supervisor):

# require 'empty' top-1 for EMPTY_REQUIRED of the last 7 before accepting new item

2.4 Decide destination

How it works on the table (real life)

- You drop one grocery item in the PICK ZONE.

- The vision node classifies it (e.g., “bottle”) and estimates the item’s pixel centroid in the camera image.

- The supervisor looks up “bottle” in a class→bin map to get the container colour and the container’s drop pose [X, Y, Z, yaw] in the robot’s base frame.

- The calibration node converts the item’s pixel position → robot base coordinates for picking.

- The IK/motion node computes a safe approach pose above the item, then executes: approach → descend → close gripper → lift.

- It then moves to the bin drop pose, opens the gripper, and retracts to a safe height.

- The arm returns home and waits for the next item.

Code you’ll need (modules + what each does)

1) config/class_to_bin.yaml — the lookup table

Stores the destination for every class, including color (for UI) and the drop pose in base coordinates:

# class_to_bin.yaml

bottle:    { color: red,    drop: [180.0,  90.0, 50.0,  0.0] }   # X,Y,Z(mm), yaw(deg)

can:       { color: yellow, drop: [180.0, -90.0, 50.0,  0.0] }

detergent: { color: purple, drop: [ 60.0, -90.0, 50.0, 90.0] }

pulses:    { color: blue,   drop: [-60.0,  90.0, 50.0, 90.0] }

seafood:   { color: green,  drop: [-60.0, -90.0, 50.0, 90.0] }

vegetable: { color: orange, drop: [ 60.0,  90.0, 50.0, 90.0] }

Load it with a tiny helper:

# utils/binmap.py

import yaml

def load_binmap(path="config/class_to_bin.yaml"):

with open(path) as f: return yaml.safe_load(f)

def get_drop_pose(binmap, klass): return binmap[klass]["drop"]

def get_color(binmap, klass):     return binmap[klass]["color"]

2) calibration/handeye.json + calib.py — pixel→base conversion

You’ll calibrate once, then save the transform(s).

If the table is planar (recommended)

Use a homography H_img2base to map image pixels (u,v) to table XY (mm). Z is the table height.

# calib.py

import numpy as np

class PlanarMapper:

def __init__(self, H_img2base, table_z_mm):

self.H = np.array(H_img2base).reshape(3,3)

self.table_z = table_z_mm

def pix_to_base(self, u, v):

p = np.array([u, v, 1.0])

q = self.H @ p                 # homogeneous mapping

x = q[0]/q[2]

y = q[1]/q[2]

z = self.table_z

return float(x), float(y), float(z)

# load from calibration file you saved earlier

# H_img2base can be computed from 4+ point correspondences when you calibrate the pick zone.

If you prefer full hand–eye

Store T_base_cam (4×4) and use a ray-plane intersection from camera to table. (Planar homography is simpler and fast for this coursework.)

3) ik.py — approach/pick/drop poses and IK

Create target poses in base frame, then solve IK/joint angles.

# ik.py

import math

def make_approach_pose(x, y, z, yaw_deg, dz=40.0):

# approach is dz mm above pick/drop

return (x, y, z + dz, yaw_deg)

def vertical_pick_sequence(x, y, z, yaw):

# returns a list of waypoints: approach -> pick -> lift

app = make_approach_pose(x, y, z, yaw, dz=40.0)

pick = (x, y, z, yaw)

lift = (x, y, z + 60.0, yaw)

return [app, pick, lift]

# IK wrapper (example; replace with Yahboom SDK or your solver)

def solve_ik(pose):

# pose: (x,y,z,yaw_deg)

# return joint angles (deg or rad) using the DOFBOT SDK or your own solver

# raise ValueError if no solution (obstacle/limits)

...

4) dofbot_api.py — motion & gripper control

A thin wrapper around Yahboom DOFBOT Python SDK.

# dofbot_api.py

import time

class Dofbot:

def __init__(self):

# init serial/SDK, set speed limits, enable torque, etc.

...

def home(self):

# known safe "ready" posture

...

def move_joints(self, joints, speed=30):

# send joint positions to arm; block until arrived

...

def move_pose(self, pose, speed=25):

# compute IK -> joints -> move

joints = solve_ik(pose)

self.move_joints(joints, speed)

def gripper(self, open=True):

# control gripper servo

...

def safe_vertical(self, waypoints, speed=25):

# ensure up/down first to avoid sweeping collisions

for p in waypoints: self.move_pose(p, speed)

5) supervisor.py — the pick & place logic (glue)

This is the state machine that listens to the vision node and executes the sequence.

# supervisor.py

from utils.binmap import load_binmap, get_drop_pose, get_color

from calib import PlanarMapper

from dofbot_api import Dofbot

from ik import vertical_pick_sequence

BINMAP = load_binmap()

MAPPER = PlanarMapper(H_img2base=..., table_z_mm=10.0)  # load your H from file

SAFE_CARRY_Z = 120.0

CARRY_YAW    = 0.0

def pick_and_place(dof, vision_msg):

"""

vision_msg = { "class":"bottle", "conf":0.86, "centroid_px": [u,v] }

"""

klass = vision_msg["class"]

u, v  = vision_msg["centroid_px"]

# 1) Decide destination

drop_x, drop_y, drop_z, drop_yaw = get_drop_pose(BINMAP, klass)

# 2) Pixel -> base for pick

x, y, z = MAPPER.pix_to_base(u, v)

# 3) Plan pick (vertical approach, then lift to carry)

pick_pts = vertical_pick_sequence(x, y, z, CARRY_YAW)

# 4) Execute pick

dof.safe_vertical([pick_pts[0]])      # approach

dof.safe_vertical([pick_pts[1]])      # descend

dof.gripper(open=False)               # close

dof.safe_vertical([pick_pts[2]])      # lift

# 5) Carry to safe height above bin

carry = (pick_pts[2][0], pick_pts[2][1], SAFE_CARRY_Z, CARRY_YAW)

dof.move_pose(carry)

# 6) Place: approach above bin, descend to drop, open, retract

place_app, place, place_lift = vertical_pick_sequence(drop_x, drop_y, drop_z, drop_yaw)

dof.safe_vertical([place_app, place]) # approach + descend

dof.gripper(open=True)                # release

dof.safe_vertical([place_lift])       # retract

# 7) Return home (ready for next)

dof.home()

You’ll call pick_and_place() whenever the vision node publishes a confirmed detection (your earlier voting logic). The centroid comes from the detector/classifier ROI; with classification-only, you can fix the centroid to the ROI center (since the item is placed in the middle), or compute it from a simple segmentation in the ROI if you want a small XY correction.

6) vision_pub.py — publish confirmed detections

(You already have this logic: temporal voting, presence, empty handling.)
Publish with ZeroMQ/Redis/ROS-like messages:

# when confirmed:

publish({"class": top_name, "conf": top_conf, "centroid_px": [cx, cy]})

7) calibrate_camera.py — one-time procedure to get H

- Place a printed ArUco/AprilTag board in the PICK ZONE.

- Click or auto-detect 4+ reference points; for each, jog the arm to touch the point and record base XY.

- Compute homography H from pixels→base and save it.

# sketch for computing H

import cv2, numpy as np

# pts_img: Nx2 pixel coords, pts_base: Nx2 mm coords

H, _ = cv2.findHomography(pts_img, pts_base, method=cv2.RANSAC)

# save H.tolist() to JSON

Putting it together (run order)

- Calibrate once → save H_img2base and table Z.

- Start vision_pub.py (classification, presence voting).

- Start supervisor.py → it listens for confirmed items and calls pick_and_place().

- If something goes wrong: E-stop, then re-home and resume.

Perfect, Yukta — this is where your robot pipeline orchestration comes in.
Let’s make it very clear how all these scripts will connect and run step-by-step on the Pi in real life.

2.5 “Brain” and “Body” Split

Your system has two main sides:

| Side | Script(s) | Role |
| --- | --- | --- |
| 🧠 Vision & Decision (Brain) | vision_pub.py → publishes class & centroid | Sees the grocery, classifies it, decides “what and where” |
| 💪 Manipulation (Body) | supervisor.py, dofbot_api.py, ik.py, calib.py | Executes the pick–place motion based on that info |

These run together — often in two processes (or threads) connected via a simple message (ZeroMQ, socket, or shared file).

⚙️ System Start-Up Order (Real-Life Workflow)

Step 1 — Boot & calibrate

You power up the DOFBOT Pi.

- Run calibration once (calibrate_camera.py) if table setup moved.

  - Saves handeye.json or H_img2base.npy.

  - This file will be loaded automatically later.

Step 2 — Start Vision Node

python3 vision_pub.py

- Opens the PiCam.

- Crops the PICK ZONE ROI.

- Runs YOLOv8n-cls every frame (≈ 10–15 FPS).

- Does temporal voting (5/7 frames rule).

- When stable, publishes a JSON message like:

- { "class": "bottle", "conf": 0.87, "centroid_px": [162, 118] }

- Then waits until the PICK ZONE is empty again before re-arming.

While running, it’s basically a live “eye” that keeps reporting confirmed groceries.

Step 3 — Start Supervisor (main controller)

python3 supervisor.py

- Subscribes/listens to messages from vision_pub.py.

- When it receives a new confirmed item:

  - Reads "class" and "centroid_px".

  - Uses calib.py to convert pixel → base coordinates.

  - Loads class_to_bin.yaml to find the drop pose for that class.

  - Calls pick_and_place() (which uses ik.py + dofbot_api.py).

Step 4 — Pick–Place Sequence (inside supervisor)

Here’s the micro-timeline that happens automatically once a grocery is detected:

| Order | Action | Code Involved |
| --- | --- | --- |
| 1️⃣ | Decide destination (read YAML, get XYZyaw) | supervisor.py, binmap.py |
| 2️⃣ | Compute pick coordinates (pixel→base) | calib.py |
| 3️⃣ | Plan safe waypoints (approach/pick/lift) | ik.py |
| 4️⃣ | Execute moves (home→approach→pick→lift→bin→drop→home) | dofbot_api.py |
| 5️⃣ | Release & retract | dofbot_api.py |
| 6️⃣ | Log cycle + cooldown | supervisor.py |

All those steps happen sequentially within the pick_and_place() function.

Step 5 — End or loop

- After one cycle, the robot returns to home pose.

- Supervisor waits for the next message from Vision.

- Vision re-arms once the zone is empty again.

- Loop repeats automatically.

You can stop either script with Ctrl +C or add a GUI “Stop” button later.

🧠💬 Communication Between Scripts

Simplest option on the Pi: ZeroMQ or Python multiprocessing.Queue.

🧾 Process Summary (Real-Life Loop)

[Operator presses Start]

│

├── vision_pub.py  (camera → YOLO → confirm class)

│       │

│       └── publishes {class, centroid_px}

│

└── supervisor.py

│

├── read message

├── map class → bin pose

├── convert pixel → base coords

├── compute IK + waypoints

├── move arm: approach → pick → carry → drop → home

└── signal done → ready for next